Pietenpol Aircamper

# Operations Manual

## Section 1

## Performance  -  Specifications

Maximum Speed – sea level			100 mph

75% power 3000 ft.				  70 mph

55% power 3000 ft.				  55 mph

Maximum Range  3000 ft

Range with 45 minutes reserve

Rate of Climb at Sea Level			400 ft per min.

Takeoff Performance

Ground Roll -	Sod				450 ft

Ground Roll – Pavement			400 ft.

Total Distance over 50 ft obstacle		2,500 ft.

Landing performance

Ground Roll					400 ft.

Distance over 50 ft. obstacle

Stall speed

Power off					29 mph

Maximum Weight				1150 lbs.

Empty Weight					  682 lbs.

Maximum Useful Load

Fuel Capacity

Total						  17 gal.

Wing tank					  13 gal.

Nose tank					    4 gal.

Wing Loading

Power Loading

Oil Capacity					    4 Qts.

Engine	   Continental				 A-65-8

65 hp at 2100 rpm

Propeller Fixed Pitch				Sensenich 72CK42

## Descriptive Data

### Engine

Number of engines:  1

Engine Manufacturer:  Continental

Engine Model Number:  A 65-8

Engine Type:  Normally apirated, direct - drive, air-cooled, horizontally – opposed,

Carburetor equipped, four cylinder engine with      cu. In. displacement.

Horsepower Rating and engine speed:  65 hp @         rpm/

### Propeller

Propeller Manufacturer:	Sensenich

Propeller  Model Number	72CK42

Number of Blades		2

Propeller Diameter		72 inches

Propeller Type			Fixed Pitch

Propeller Material		Wood

### Fuel

Approved Fuel Grades

100 LL Grade Aviation Fuel (Blue)

80/87  Auto Fuel ( No Alchohol)

### Oil

Specifications

Sump				4 Qts.

Below 40 degrees F		SAE 20

Above 40 degrees F		SAE 40

Note When temperaturers overlap, use the lighter grade of oil.

### Maximum Weight

Maximum Ramp Weight	 1150 lbs.

Maximum Takeoff  Weight	 1150 lbs.

Maximum Landing Weight 	 1100 lbs.

### Standard Airplane Weight

Standard Empty Weight	682 lbs.

Maximum Useful Load

Baggage Space

Wing Locker			10 lbs.

### Specific Loadings

Wing Loading:

Empty				10.4

Maximum			17.69

Power Loading:

Empty				4.7

Maximum			7.18

## Section 2

## Limitations

## Introduction

Section 2 includes operating Limitations, instrument Markings and basic Placards necessary for the safe operation of the airplane, its engine, standard systems and  standard equipment.  Observance of these operating limitations is required by the Federal Aviation Administration regulations.

### Airspeed Limitations

Vne		Never Exceed Speed		100 mph.

Vno		Maximum Structural 		  80 mph

Cruising Speed

Va		Maneuvering Speed

1150 lbs

Vs		Stall Speed			  29 mph

### Marking

Green Arc	Normal Operating Range	29-80 mph

Yellow Arc	Smooth Air Operating		80-100 mph

Power Plant Limitations

Engine Manufacturer: Continental

Engine Model Number: A 65-8

Maximum Power: 65 BHP rating

Engine operating Limits for takeoff and continuous Operations:

Maximum Engine Speed  2300 rpm

Note

The static RPM range at full throttle ( carburetor heat off ) is 2000 RPM.

Maximum Oil Temperature:  225 degrees.

Oil Pressure

Minimum				10 psi

Cruising				30-40 psi

Maximum				85 psi

Maximum Cylinder head Temperature	550 degrees F

Appropriate Maximum Oil Consumption	¾ Pint per hour

Fuel  Grade (see fuel limitations)

Oil Grade:

Below 40 degrees F				SAE 20

Above 40 degrees F				SAE 40

Propeller Manufacturer 		Sensenich

Propeller Model Number		72CK42

Propeller Diameter			72 inch

### Power Plant Instrument Markings

Tachometer

Normal Operating		1500-2300 rpm

Oil Temperature			100 – 225 Degrees F.

Oil Pressure

Minimum			20 psi

Maximum			85 psi

Fuel Quantity

Wing tank			13 gal.

Nose tank			  4 gal.

Weight Limits

Maximum Ramp Weight 		1150 lbs.

Maximum Takeoff Weight		1150 lbs.

Maximum Landing Weight		1100 lbs.

Maximum weight in wing		   10 lbs.

Baggage locker

### Center of Gravity Limits

Center of Gravity range

Forward			   15 inches aft of datum

Rear				   20 inches aft of datum

Reference datum:  Leading edge of wing

### Maneuvering Limits

This airplane is certified in the Experimental category.  The experimental Category is applicable to aircraft intended for               operations.  These include maneuvers incidental to normal flying.  Aerobatic maneuvers, including spins in this aircraft are not permitted.

Flight Load  Factor Limits

Flight Load Factors - Maximum takeoff weight 	1150 lbs.

### Kinds of Operating Limits

This aircraft is equipped for day VFR flight.  FAR Part 91 establishes the minimum required instrumentation and equipment for these operations.  The referance to types of flight operations on the operating limitations placard reflects equipment installed at the time of Airworthiness Certificate issuance.

Flight into known icing conditions is prohibited.

### Fuel Limitations

2 tanks

Total fuel 17 gallons

Wing tank			13 gal.

Nose tank			  4 gal.

Fuel in nose tank is not intended for use in any normal operations.  Nose tank is intended as a constant feed in maneuvers where fuel supply may be interrupted.

Placards

The following information must be displayed in the form og composite or individual placards.

On the fuel shutoff  handle.

Fuel Shutoff – Pull

Next to wing tank fuel shutoff on Cabane Strut

ON and arrow showing direction of valve operation.

Near fuel tank filler

80 / 87 Min Grade Gasoline

Fuel Capacity 17 gal.

On Wing baggage door

10 lbs. Max Capacity

A calibration card must indicate the accuracy of the compass in 30 degree increments.

On Oil filler cap

Oil    1 Gal.

Under fuel primer

Primer

On Passenger front panel

PASSENGER WARNING

THIS AIRCRAFT IS AMATUER BUILT AND

DOES NOT COMPLY WITH THE FEDERAL SAFETY

REGULATIONS FOR “STANDARD AIRCRAFT”.

Section 3

Emergency Procedures

### Introduction

Section 3 provides checklist and amplified procedures for coping with emergencies that may occur.  Emergencies that caused by airplane or engine malfunctions are extremely rare if proper preflight inspections and maintenance are practiced.  Enroute weather emergencies can be minimized or eliminated by careful flight planning and good judgement when unexpected weather is encountered. However, should an emergency arise, the basic guidelines described in this section should be considered and applied as necessary to correct the problem.

### Airspeeds for Emergency Operation

Engine Failure after takeoff		65 mph

Maneuvering speed

1150 lbs			80 mph

Maximum Glide			55 mph

Precautionary landing

With engine power		55 mph

Landing without engine Power	55 mph

### Operational Checklists

Procedures in the operational Checklists portion of this section shown in Bold – Faced type are immediate – action items which should be committed to memory.

#### Engine Failures

### Engine Failure During Takeoff Roll

- Throttle --idle

- Brakes  -- apply

- Ignition Switch – Off

### Engine Failure immediately after takeoff

- Airspeed – 65 mph

- Fuel Selector Valve – Pull to Off

- Ignition Switch – Off

### Engine Failure During Flight

- Airspeed – 55 mph

- Carburetor heat – On

- Fuel Selector Valve – On

- Magneto Switch – Both

- Primer – In and locked

Forced Landings Without Engine Power

- Seat belts and shoulder harnesses – Secure

- Airspeed –55 mph

- Fuel shutoff valve – Pull to Off

- Magneto Switch – Off

- Touchdown  --55 mph - tail low

- Brakes – Apply

### Forced Landing with Engine Power

- Seat belts and Shoulder harnesses -- Secure

- Airspeed -- 55 mph

- Select field – Fly over, noting terrain and obstructions.

- Turn off avionics and beacon switches.

- Airspeed – 55 mph for touchdown

- Touchdown – tail low

- Magneto Switch – Off

- Apply Brakes

## Fires

### During Start on Ground if engine starts

- If engine starts  -- 1700 rpm for a few minutes.

- Engine shutdown – inspect for damage

### If engine fails to start

- Exit aircraft immediately

- Throttle  -- Full open

- Fuel shutoff – Pull to close

- Magneto Switch – Off

- Fire extinguisher – Obtain – have ground attendant obtain if not installed

- Fire damage – Inspect, repair damage or replace damaged compoments before conducting another flight.

### Engine Fire in Flight

- Fuel Shutoff Valve – Pull to Off

- Avionics and Beacon Switches – Off

- Airspeed – 80 mph ( if fire does not extinguished, increase glide speed to find an airspeed which will provide an incombustable mixture.

- Forced Landing – EXECUTE ( as described in Emergency Landing Without Power )

### Landing with a Main Flat Tire

- Approach – Normal

- Touchdown – On Good Tire First, hold airplane off flat tire as long as possible.

###### Section 4   Normal Procedures

### Introduction

Section 4 provides checklists procedures for normal operation.

### Speeds for Normal Operation

Unless otherwise noted, the following speeds are based on a maximum weight of 1150 pounds and may be used for any lesser weight.  However, to achieve the performance  specified in Section 5 for takeoff distance, the speed appropriate to the particular weight must be used.

Takeoff

Normal Climb Out				65 mph

Short Field Takeoff				55 mph

Enroute climb

Best rate of Climb				65 mph

Landing Approach

Normal Approach				65 mph

Touchdown speed				55 mph

Maximum Turbulent Air Penetration Speed 		80 mph

Maximum Crosswind Velocity			10 mph

## Checklist Procedures

## Preflight Inspection

### Introduction

Visually inspect the airplane for general condition during the walk-around inspection.  Use of the refueling steps will simplfy access to the upper wing surfaces for visual checks and re-fueling operations.  In cold weather, remove even small accumulations of  frost, ice or snow from wing, tail and control surfaces.Also, make sure that control surfaces contain no internal accumulations of ice or debris.

## Preflight Inspection

### Cabin

- Pilot Operating Handbook in airplane

- Airplane Weight and Balance checked

- Wheels Chocked

- Control stick lock removed

- Magneto switch – Off

- Fuel quantity indicator – check Quantity   Airplane must be level to read quantity

- Fuel Valve – ON

- Wing Baggage door Secured

### Right Wing

- Check fabric condition

- Check aileron freedom of movement

- Check aileron hinge screws

- Check aileron cable fittings

- Check pitot tube  on leading edge of Wing

- Remove gust lock if installed

- Wing tie down – disconnect

- Check lift strut condition and safety wires

### Nose

- Engine Oil Dipstick filler Cap – Check oil level, then check dipstick filler cap

Secure.  Do not operate with less than 4 quarts.

- Fuel strainer drain on gascolator – sample fuel

- Propeller – Check leading and trailing edges for nicks and damage.  Check propeller bolt safety wires

- Check inside of cooling eyebrows for obstructions

- Check air filter

### Left Wing and Fuel fill

- Fuel fill cap secure

- Verify fuel level

- Check fabric condition

- Check aileron freedom of movement

- Check aileron hinge screws

- Check aileron cable fittings

- Remove gust lock if installed

- Wing tie down – disconnect

- Check lift struts and safety wires

### Fuselage

- Check landing gear, tire pressures, condition of bungees, brakes and legs.

- Check condition of fabric

- Check condition of access plates under fuselage

- Check tail wheel assembly, condition of  control cables

- Check rudder and stabilizer. Condition of control cables and of hinges.

### Before Starting Engine

- Wheels chocked

- Preflight inspection – complete

- Passenger briefing – complete

- Seat belts and shoulder harnesses Adjust and Lock

- Fuel valve  -- ON

##### WARNING

When pulling the propeller through by hand, make sure that the Magneto switch is Off.

### Starting Engine

- Prime – As required  ( 2 – 6 strokes, none if engine is warm)

- Carburetor Heat – Cold

- Throttle Closed

- Propeller area clear

- Magneto Switch – ON – Both

- Engine starts by hand prop.

- Check oil pressure

- Establish idle speed of 650-800 rpm

### Before Takeoff

- Seat belts and shoulder harnesses secure.

- Flight controls – free and correct

- Flight instruments Check and Set

- Fuel quantity – Check

- Primer – In and Locked

- Fuel Valve – Re- check open

- Throttle 1400 rpm

  - Magneto check rpm drop should not exceed 50 rpm on either magneto or 50 rpm difference between magnetos.

  - Carburetor Heat – Check for rpm drop

  - Engine instruments – Check

- Throttle 650-800 rpm

- Strobe Light – as desired

### Takeoff

- Carburetor Heat – Cold

- Throttle – Full Open

- Elevator Control – lift tail at 23 mph

- Climb Speed – 65 mph

### Short Field Takeoff

- Carburetor Heat – Cold

- Brakes –Apply

- Throttle – Full Open

- Brakes Release

- Lift tail as soon as Possible

- Climb at 55 mph until obstruction is cleared

- Climb at 65 mph

## Landing

### Normal Landing

- Airspeed 65 mph

- Add carburetor heat when engine speed less than 1700 rpm

- Airspeed 55 mph on short final

### Balked Landing

- Throttle – Full Open

- Carburetor Heat – Cold

- Climb speed – 55mph

### After Landing

1.  Carburetor Heat – Cold

### Securing Airplane

- Avionics Switches Off

- Magneto Switch Off

- Control locks – Install

- Fuel Valve – Pull to shutoff

### Cold Weather Operation

Special consideration should be given to the operation of the airplane fuel system during the winter season or prior to any flight in cold temperatures.  Proper preflight draining of the fuel system is especially important and will eliminate any free water accumulation.

Cold weather often causes conditions which require special care during airplane operations. Even small accumulations of frost, ice or snow must be removed from wings, tail and control surfaces to assure satisfactory flight performance and handling.

Starting

Prior to starting on cold mornings, it is advisable to pull the propeller through several times by hand  to “break loose” or “limber the oil.

Caution

Before Pulling through the Propeller through, Verify the Magneto switch is OFF.

### Starting

- Wheels chocked

- Magnetos -- OFF

- Prime as required – ( 2 –6 strokes)

- Throttle – Closed

- Magneto switch – ON

- Area Clear

- Hand prop engine

Section 5     Performance

Cruise conditions

Total Range –no reserve

Presssure Altitude				3000 ft.

Expected wind					0

Temperature					20 degrees Centigrade

Landing Conditions

Field pressure altitude				1000 ft

Temperature 					25 degrees Centigrade

Field length					1500 ft

### Fuel Required

The total fuel required may be estimated by using the basic performance information for the Continental A-65-8 engine.

Figure 3 Continental Engin Manual

Figure   Temperature Conversion Chart

Place conversion chart Here

Section 6    Weight and Balance

Place weight  and balance calculations here

###### Section 7

Airplane & Systems Descriptions

#### Parts and Equipment List

Continental A-65-8

Magnetos			Slick

Exhaust			Luscomb

Propeller			Sensenich 72CK x 42 wood

Battery				31 amp hour sealed gell 12 v.

Gascolator			ACS

Fuel filter

Engine eyebrows		Homemade from pattern of J-3

Compass			Airpath

Oil pressure Guage		2 ¼”

Oil temperature Guage	2 ¼”

Magneto Switch		A-7

Tachometer

Altimeter

Airspeed Indicator

Turn coordinater

Engine Primer			ACS

GPS				Garmin GPS II

Strobe light

Pilot and Passenger Seatbelt

Pilot and Passenger wind screen

2 – 12 volt outlets

- Throttle

- Carburetor heat

- Fuel shutoff

- Magneto Switch

- Fuel primer

- Altimeter

- Compass

- Airspeed

- Turn coordinater

- Tachometer

- Oil temperature

- Oil pressure

- Elt switch

- Strobe switch

- 12 volt outlets

- brake resoiver

- Brake handle

- Wing fuel shutoff