Table of MC&MK oil dilution

- during diluting ,the rotational frequency is 1600 rpm

- when the rotational frequency reaches 1600 rpm before diluting, the fuel pressure will be 0.35 kg/cm2

- after diluting , when the quantity of  oil in the tank is 15kl, the engine’s rotational frequency will keep 1600rmp for 4 to 5 minutes. If the there is a little of oil in the tank, the frequency will keep for 3 minutes.

| Working time of engine after last diluting (hour——minute) | Quantity of oil in the tank before diluting (kilolitre) |  |  |
| --- | --- | --- | --- |
|  | 15 | 12 | 9 |
|  | Time of keying on the diluting switch(minute-second) |  |  |
| 0——15 | 1——43 | 1——28 | 1——14 |
| 0——30 | 2——44 | 2——21 | 1——57 |
| 0——45 | 3——27 | 2——58 | 2——29 |
| 1——00 | 3——41 | 3——17 | 2——46 |
| Non-diluted oil | 4——36 | 3——47 | 3——10 |