            TABLE WITH TORQUE REQUIREMENTS AT M-14P ENGINE

	Nomenclature
	Thread size
	Torque

	
	
	Kgfm
	In. Lbs.

	Propeller governor hold nuts
	M8
	1.3-1.7
	113-147

	Drain tank hold nuts and bolts
	M7
	.95-1.2
	82-104

	Signaler  filter hold nuts
	M5
	.45-.6
	39-52

	Cylinder hold down nuts
	M10
	3.5-3.8
	304-330

	Magnetos hold nuts
	M8
	1.3-1.7
	113-147

	Generator adapter hold nuts
	M10
	2-2.5
	174-217

	AK-50A air compressor hold nuts
	M6
	.7-.9
	61-78

	Oil pump hold nuts
	M7
	.95-1.2
	82-104

	Fuel pump hold nuts
	M7
	.95-1.2
	82-104

	Vacuum pump drive hold nuts
	M8
	1.3-1.7
	113-147

	Air distributor hold nuts
	M5
	.45-.6
	39-52

	Air start valves
	M14
	2.5-3
	217-260

	Carburetor hold nuts
	M8
	1.3-1.7
	113-147

	Spark plugs
	M14
	2.5-3
	217-260


