YAK 52 N288Y
Serial Number 888714
Manufactured 1988- US Entry 1993
 Conditional Inspection – 2003 and Forward
Aubrey Price A&P 458709596

Time at Last Entry 411.0 Hrs.
	Item
	OK
	Time
	Date
	Discrepancies

	Propeller
	 
	329.1
	August 19,2002
	Overhauled Hub by Gesoco and static balance  (  August 19,2002)

	   Blades
	x
	329.1
	 
	Inspected

	   Bearings
	x
	354.0
	January25, 2004
	Inspected

	   Seals
	x
	354.0
	January25, 2004
	Inspected

	   Prop Hub
	x
	354.0
	January25, 2004
	Hub moved, inspected, seals inspected, checked bearings, lubed slider, reinstalled and safety wired.

	Check Blade Angle
	x
	351.0
	November 17,2003
	Set at 14 Degrees

	Refinish and paint Prop
	X
	356.0
	Sept 19,2004
	Complete

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	Engine   
	 
	 
	 
	 

	   Finger Screens
	x
	329.1
	August 3,2003
	Pulled, Cleaned, Inspected

	   Carburetor
	x
	329.1
	August 3,2003
	Inspected

	   Mags
	x
	329.1
	August 3,2003
	Set to factory specs.

	   Governor
	x
	329.1
	August 3,2003
	Operational Checked

	   Plugs
	x
	329.1
	August 3,2003
	Cleaned and Gapped

	   Valve Timing
	x
	338.8
	October 28,2003
	Set to factory specs.

	   Cylinders
	x
	329.1
	August 3,2003
	Cold reading = 1=76 2=75 3=71 4=72 5=75 6=75 7=74 8=77 9=76 / over 80

	   Cylinders
	x
	411.1
	April 1,2005
	Cold reading = 1=76 2=78 3=76 4=72 5=76  6=75 7=76 8=77 9=76 / over 80

	Install new exhaust gasket on 8&9
	x
	356.00
	Sept 19,2004
	 Complete

	Install new exhaust clamps on 1,2,3,5,6
	x
	411.1
	April 7,2005
	 Complete

	   Exhaust
	x
	329.1
	August 3,2003
	Inspected

	   Generator
	x
	329.1
	August 3,2003
	Inspected

	   Inlet nuts
	x
	329.1
	August 3,2003
	Inspected

	   Baffles
	x
	329.1
	August 3,2003
	Inspected

	   Cooling gills
	x
	329.1
	August 3,2003
	Inspected

	   Carb heat box
	x
	329.1
	August 3,2003
	Inspected ( Was sticking, Has been repaired)

	   Compressor
	x
	334.2
	August 17,2003
	Cleaned output valve and installed new Crush washers

	Engine Mounts
	x
	334.2
	August 17,2003
	Inspected

	Lower Drain Kit
	x
	350.1
	October 29,2003
	Installed lower cylinder  drain kit

	Carb heat Inspected
	x
	334.2
	August 17,2003
	New spring installed

	Fuel Hoses
	x
	352.0
	January 7.2004
	All fuel hoses were removed and replaced with Carl Hayes Enterprise American rubber hoses.

	Heated Pads
	x
	354.0
	January25, 2004
	Installed 2 EZ heat pads on oil cooler and one on oil sump.

	 
	 
	 
	 
	 

	Change drain kit to flex hoses
	x
	356.0
	Sept 19,2004
	New Hoses to replace hard tubing

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	Fuselage
	 
	 
	 
	 

	   Structure
	x
	329.1
	August 3,2003
	Inspected

	   Front cockpit
	x
	329.1
	August 3,2003
	Inspected

	   Instruments
	x
	329.1
	August 3,2003
	Front VSI needs repair ( Light bulbs replaced )

	   Rear cockpit
	x
	329.1
	August 3,2003
	Inspected

	   Instruments
	x
	329.1
	August 3,2003
	Inspected( Light bulbs replaced )

	   Plexiglas
	x
	334.2
	August 17,2003
	Stop drilled two small cracks *

	   Air bottles
	x
	329.1
	August 3,2003
	Charged to 50 ATM ( Small Air Leak )

	   Avionics compartment
	x
	329.1
	August 3,2003
	Inspected

	   Trim system
	x
	329.1
	August 3,2003
	Inspected

	   Control cables
	x
	329.1
	August 3,2003
	Operational Checked

	 Clean and Lube all rollers
	x
	338.8
	October 28,2003
	Cleaned and lubed

	   Radio
	x
	329.1
	August 3,2003
	Operational Checked

	   Intercom
	x
	329.1
	August 3,2003
	Operational Checked

	   Gyro Compass
	x
	329.1
	August 3,2003
	Operational Checked

	   Slaving system
	x
	329.1
	August 3,2003
	Operational Checked

	   Fuel gauge
	x
	329.1
	August 3,2003
	Operational Checked

	   Battery compartment
	x
	329.1
	August 3,2003
	Has gel cell batteries

	   Charging system
	x
	329.1
	August 3,2003
	Operational Checked

	   Front canopy
	x
	329.1
	August 3,2003
	Inspected

	   Rear canopy
	x
	329.1
	August 3,2003
	Inspected

	   Elevator roller
	x
	338.8
	October 28,2003
	Cleaned and lubed

	   Trim actuator
	x
	338.8
	October 28,2003
	Cleaned and Lubed

	   Counter balance bolts
	 
	356.0
	Sept 19,2004
	Inspected

	Replace Seat Belts ( both Cockpits )
	x
	351.0
	November 17,2003
	Install new Hooker 5 point Sport Harness in both cockpits

	Remove Russian ADF antenna and Radio
	x
	356.0
	May 7,2004
	Remove antenna and reinstall cover

	Install new Interior
	x
	356.0
	Sept 19,2004
	Complete

	Sand and repaint aircraft
	x
	356.0
	Sept 19,2004
	Complete

	Clean and Paint Interior
	x
	356.0
	Sept 19,2004
	Complete

	Remove all fairings and Inspection plates and Inspect
	x
	356.0
	Sept 19,2004
	Complete

	Install new stainless steel screws in fairings
	x
	356.0
	Sept 19,2004
	Complete

	Install new stainless steel screws in Inspection plates
	x
	356.0
	Sept 19,2004
	Complete

	Inspect all pullies and cables
	x
	356.0
	Sept 19,2004
	Complete

	Install new ADF antenna
	x
	356.0
	Sept 19,2004
	Complete

	Remove Russian ADF Box
	x
	356.0
	Sept 19,2004
	Complete

	Relocated ELT to Russian Radio Rack
	x
	356.0
	Sept 19,2004
	Complete

	Install new quick disconnect seat pins
	x
	356.0
	Sept 19,2004
	Complete

	Install new decals in both cockpits
	x
	356
	Sept 19, 2004
	Complete

	Install YAK sack 
	x
	356.0
	Sept 19, 2004
	 

	Install FOD kit in rear fuselage
	x
	352.2
	January 7,2004
	Install FOD kit

	Install New American VSI
	x
	411.0
	April 14, 2005
	Complete

	 Nose gear
	 
	 
	 
	 

	      Actuator
	x
	329.1
	August 3,2003
	Operational Checked

	       Shimmy Damper
	x
	336.2
	September1,2003
	Remove, install overhaul kit, reinstall

	      Wheel/tire
	x
	329.1
	August 3,2003
	Inspected

	       Wheel Bearing
	x
	336.2
	September1,2002
	Checking and packed

	 
	 
	 
	 
	 

	      Strut
	x
	329.1
	August 3,2003
	Inspected/ 

	       Strut         
	x
	351.1
	January 2004
	Inspected/ Added nitrogen to factory specs.

	      Up lock
	x
	329.1
	August 3,2003
	Operational Checked

	       Gear air cylinder
	x
	335.4
	August 30,2003
	Remove air fittings, lube with airtool oil, Reinstall and soap check

	 
	 
	 
	 
	 

	      Indicator
	x
	329.1
	August 3,2003
	Operational Checked

	      Inspect Brakes and Bearings
	x
	356.0
	Sept 19,2004
	Inspected

	Install new anti Fade Brake Pads
	 
	 
	 
	 

	      Inspect Switches 
	x
	356.0
	Sept 19,2004
	Inspected

	 
	 
	 
	 
	 

	      Gear Retraction Test on Jacks
	x
	356.0
	Sept 19,2004
	Inspected

	      Remove fairings and Inspect
	x
	356.0
	Sept 19,2004
	Inspected

	      Repaint Gear
	x
	356.0
	Sept 19,2004
	Inspected

	       Service strut
	x
	356.0
	Sept 19,2004
	Inflate to factory specs.

	 
	 
	 
	 
	 

	Fin/rudder      
	 
	 
	 
	 

	   Bearings
	x
	332.6
	August 3,2003
	Inspected

	   Lower bearing retainer
	x
	 x
	Sept 19,2004
	 Inspected

	   Fabric
	x
	332.6
	August 10,2003
	Repaired small area

	   cables/forks
	x
	351.1
	January 2004
	Checked

	Recover with heavy Stitts Fiber
	 
	356.0
	Sept 19,2004
	Complete

	Add Carl Hayes anti drum fix to rudder
	 
	356.0
	Sept 19,2004
	Complete

	Tail/elevators
	 
	 
	 
	 

	   Hinges
	x
	332.6
	August 10,2003
	Inspected

	   Fabric
	x
	332.6
	August 10,2003
	Inspected

	   Trim tab
	x
	332.6
	August 10,2003
	Operational Check

	Recover with Stitts Heavy Fiber
	 
	 
	 
	 

	Inspect for crack
	 
	356.0
	Sept 19,2004
	   Inspected  

	Wing-Left
	 
	 
	 
	 

	   Fuel tank
	x
	329.1
	August 3,2003
	Inspected

	   Fuel vent
	x
	329.1
	August 3,2003
	Inspected

	   scupper drain
	x
	329.1
	August 3,2003
	Inspected

	   Pitot probe
	x
	329.1
	August 3,2003
	Inspected

	   Pitot heat
	x
	329.1
	August 3,2003
	Inspected

	   Stall warning vane
	x
	351.1
	December 1,2003
	Removed and repaired ( Stall and Underspeed work correctly )

	   L.E.
	x
	329.1
	August 3,2003
	Inspected

	   Gear
	x
	3256.0
	Sept 19,2004
	 Inspected

	      strut
	x
	329.1
	August 3,2003
	Inspected

	     Strut
	X
	352.1
	January 7,2004
	Add nitrogen to factory specs.

	      actuator
	x
	329.1
	August 3,2003
	Operational Checked

	      up-lock
	x
	329.1
	August 3,2003
	Operational Checked ( Break air lines, insert Airtool oil, soap test. 

	      indicator
	x
	329.1
	August 3,2003
	Operational Checked

	      brakes
	x
	332.6
	August 10,2003
	Removed and inspected -7MM min. thickness or ears broken off

	New Brakes
	X
	369.0
	Nov 1,2004
	New Low fade brakes from Mark Jeffery

	      hose
	x
	329.1
	August 3,2003
	Inspected

	      Gear air Cylinders
	x
	335.4
	August 30,2003
	Remove air fittings, lube with airtool oil, Reinstall and soap check

	      Gear Retraction Test on Jacks
	x
	356.0
	Sept 19,2004
	Inspected

	      Remove fairings and Inspect
	x
	356.0
	Sept 19,2004
	Inspected

	      Inspect Brakes and Bearings
	x
	356.0
	Sept 19,2004
	Inspected

	      Inspect Switches 
	x
	356.0
	Sept 19,2004
	Inspected

	      Repaint Gear
	x
	356.0
	Sept 19,2004
	Inspected

	       Service strut
	x
	356.0
	Sept 19,2004
	Inflate to factory specs.

	   Aileron   
	 
	 
	 
	 

	      fabric
	x
	332.6
	August 9,2003
	Repaired minor fabric on both 

	      inbd. hinge
	x
	329.1
	August 8,2003
	Inspected

	      tip damage
	x
	329.1
	August 3,2003
	Inspected

	      push rod/bearing
	x
	329.1
	August 3,2003
	Operational Checked

	   Flap
	 
	 
	 
	 

	      TE
	x
	329.1
	August 3,2003
	Inspected

	      Links
	x
	335.4
	August 30,2003
	Inspected( Rollers cleaned and lubed, grease fitting serviced

	      Pushrod
	x
	329.1
	August 3,2003
	Inspected

	      Switch
	x
	329.1
	August 3,2003
	Inspected

	Recover with Heavy Stitts Fiber
	 
	 
	 
	Complete

	Wing-Right
	 
	 
	 
	 

	   Fuel tank
	x
	329.1
	August 3,2003
	Inspected

	  Repair  Fuel tank leak
	x
	356.0
	Sept 19,2004
	Complete

	   Fuel vent
	x
	329.1
	August 3,2003
	Inspected

	   scupper drain
	x
	329.1
	August 3,2003
	Inspected

	   L.E.
	x
	329.1
	August 3,2003
	Inspected

	   Gear
	 
	 
	 
	 

	     Strut
	X
	352.1
	January 7,2004
	Add nitrogen to factory specs.

	      strut
	x
	329.1
	August 3,2003
	Inspected

	      actuator
	x
	329.1
	August 3,2003
	Operational Checked

	      up-lock
	x
	329.1
	August 3,2003
	Operational Checked ( Break air lines, insert Airtool oil, soap test. 

	      indicator
	x
	329.1
	August 3,2003
	Operational Checked

	      brakes
	x
	332.6
	August 10,2003
	Removed and inspected  -  7 MM min. thickness or ears broken off

	
	
	
	
	
	


	New Brakes
	    X
	369.0
	Nov 1,2004
	New Low fade brakes from Mark Jeffery


	      hose
	x
	329.1
	August 3,2003
	Inspected

	      Gear air Cylinders
	x
	335.4
	August 30,2003
	Remove air fittings, lube with airtool oil, Reinstall and soap check

	      Gear Retraction Test on Jacks
	x
	356.0
	Sept 19,2004
	Complete

	      Remove fairings and Inspect
	x
	356.0
	Sept 19,2004
	Complete

	      Repaint Gear
	x
	356.0
	Sept 19,2004
	Complete

	       Service strut
	x
	356.0
	Sept 19,2004
	Inflate to factory specs.

	      Inspect Brakes and Bearings
	x
	356.0
	Sept 19,2004
	Complete

	      Inspect Switches 
	x
	356.0
	Sept 19,2004
	Complete

	      tire
	x
	329.1
	August 3,2003
	Inspected

	   Aileron   
	 
	 
	 
	 

	      fabric
	x
	332.6
	August 9,2003
	Repaired minor fabric on both 

	      inbd. hinge
	x
	329.1
	August 8,2003
	Inspected

	      tip damage
	x
	329.1
	August 3,2003
	Inspected

	Recover with Heavy Stitts Fiber
	x
	356.0
	Sept 19,2004
	Complete

	      push rod/bearing
	x
	329.1
	August 3,2003
	Operational Checked

	   Flap
	 
	 
	 
	 

	      TE
	x
	329.1
	August 3,2003
	Inspected

	      Links
	x
	335.4
	August 30,2003
	Inspected( Rollers cleaned and lubed, grease fitting serviced

	      Pushrod
	x
	329.1
	August 3,2003
	Inspected

	      Switch
	x
	329.1
	August 3,2003
	Inspected

	      Switch
	X
	356.0
	May 7,2004
	Adjust Switch

	Flight test
	 
	 
	 
	 

	   Engine Acceleration
	x
	329.1
	August 3,2003
	Operational Checked

	   Engine RPM
	x
	329.1
	August 3,2003
	Operational Checked

	   Elevator trim
	x
	329.1
	August 3,2003
	Operational Checked

	   Cruise speed
	x
	329.1
	August 3,2003
	Operational Checked

	   Roll trim - cruise
	x
	329.1
	August 3,2003
	Operational Checked

	   Roll trim - pattern
	x
	329.1
	August 3,2003
	Operational Checked

	   Pitch response
	x
	329.1
	August 3,2003
	Operational Checked

	   Inverted oil
	x
	339.5
	October 5,2003
	Operational Checked by Aubrey Price ATP A&H 1813638

	   Inverted fuel
	x
	339.5
	October 5,2003
	Operational Checked by Aubrey Price ATP A&H 1813638

	   Stall behavior
	x
	329.1
	August 3,2003
	Operational Checked

	   CHT
	x
	329.1
	August 3,2003
	Operational Checked

	   Oil temp
	x
	329.1
	August 3,2003
	Operational Checked

	   Flight instruments
	x
	329.1
	August 3,2003
	Operational Checked

	   Gear indicators
	x
	329.1
	August 3,2003
	Operational Checked

	   Engine smoothness
	x
	329.1
	August 3,2003
	Operational Checked

	   General
	x
	329.1
	August 3,2003
	Operational Checked

	   Brakes
	x
	329.1
	August 3,2003
	Operational Checked 

	   Engine Acceleration
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Engine RPM
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Elevator trim
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Cruise speed
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Roll trim - cruise
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Roll trim - pattern
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Pitch response
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Inverted oil
	x
	357.0
	Sept 18,2004
	Operational Checked by Aubrey Price ATP A&H 1813638

	   Inverted fuel
	x
	357.0
	Sept 18,2004
	Operational Checked by Aubrey Price ATP A&H 1813638

	   Stall behavior
	x
	357.0
	Sept 18,2004
	Operational Checked

	   CHT
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Oil temp
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Flight instruments
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Gear indicators
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Engine smoothness
	x
	357.0
	Sept 18,2004
	Operational Checked

	   General
	x
	357.0
	Sept 18,2004
	Operational Checked

	   Brakes
	x
	357.0
	Sept 18,2004
	Operational Checked 

	   Inverted oil
	x
	357.0
	Sept 18,2004
	Operational Checked by Aubrey Price ATP A&H 1813638

	   Inverted fuel
	x
	357.0
	Sept 18,2004
	Operational Checked by Aubrey Price ATP A&H 1813638

	 
	 
	 
	 
	 

	Remove and Overhaul Compressor
	x
	356.0
	Aug19,2004
	Complete

	Electric/air start valve
	x
	334.2
	August 24,2003
	Checked and lubed with air tool oil.

	 
	 
	 
	 
	 

	Inspect Main Air Tank
	 
	 
	October 28,2003
	Removed, cleaned and inspected

	Inspect Reserve Air Tank
	 
	 
	October 28,2003
	Removed, cleaned and inspected

	Inspect Main Air Tank
	x
	356.0
	Sept 15,2004
	Removed, cleaned and inspected ( pressure to 150% )

	Inspect Reserve Air Tank
	X
	356.0
	Sept 15,2004
	Removed, cleaned and inspected ( pressure to 150% )

	 Leak Test System  
	x
	334.2
	August 28,2003
	Holds main and Reserve pressure with Valves off.

Leaks with valves on.

	 Leak Test System  
	x
	356.0
	Sept 15,2004
	Holds main and Reserve pressure with Valves off.

No Leaks

	         Bubble test for system leaks with Valves off
	x
	335.4
	August 28,2003
	No leaks with valves off

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	         Test system for leaks with main air valves on
	x
	335.4
	August 29,2003
	Bubble testing hoses, connections and cylinders 

	One Way valve to SNOT bottle
	x
	350.0
	 
	Replace Seal and reseal Body

	One Way valve to SNOT bottle
	x
	356.0
	Sept 15,2004
	Installed new One way Valve from Carl Hayes

	 
	 
	 
	 
	 

	Air Line from Dist. Block to firewall
	 
	 
	 
	Replace with new line and reflare

	L between Snot bottle and One way valve
	x
	345.0
	Oct 26,2003
	Found crack and replaced fitting

	Cowling
	 
	 
	 
	 

	Lower drain Hole
	x
	356.0
	January 24, 2004
	Add doubler to lower drain exit hole

	Repair Lower drain broken tube
	x
	356.0
	Sept 19,2004
	Complete

	Install new rear cowling pins from Carl Hayes
	x
	356
	Sept 19,2004
	complete

	Install chafe tape on cowlin
	x
	356.0
	Sept 19,2004
	Complete

	Repair rub area on cowling with .0020 and rivets
	x
	356.0
	May 7, 2004
	Complete

	 Install Cowling Chafe tape     
	 x
	 398.0
	 Feb 2005
	Complete 

	Future Mods
	 
	 
	 
	 

	 
	 
	 
	 
	 

	     Gear Horn                 
	 
	 
	Future
	Sound gear horn if flaps or down and gear is up

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	Item
	OK
	Time
	Date
	Discrepancies

	Paperwork
	 
	 
	 
	 

	   Engine log (US)
	x
	329.1
	August 3,2003
	Present

	   Engine log (RUS)
	 
	329.1
	August 3,2003
	Present

	   Airframe log (US)
	x
	329.1
	August 3,2003
	Present

	   Airframe logs (RUS)
	 
	 
	August 3,2003
	Present

	   Operating Limitations
	x
	329.1
	August 3,2003
	Present

	   C of A
	x
	329.1
	August 3,2003
	Present

	   Last Annual
	x
	329.1
	August 3,2003
	Conditional Inspection - Aubrey Price A&P 458709596

	   Last Annual
	x
	356.0
	Sept 19,2004
	Conditional Inspection - Aubrey Price A&P 458709596

	   Registration certificate
	x
	329.1
	August 3,2003
	New Certificate from FAA has not arrived. Temp is in aircraft

	   Weight and Balance
	x
	329.1
	August 3,2003
	Weight and balance info from original delivery in 199

	Weight and Balance
	x
	360.0
	Sept 24,2004
	New W&B by Robert Q. Steinman, A&P, Ph.D. 1501752

FAA Certified Electronic Scales

	   POH
	x
	329.1
	August 3,2003
	Checked

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	New Operational Limits Issued
	x
	338.5
	October 2,2003
	Dallas FAA Office

	New Airworthiness Certificate Issued
	x
	338.5
	October 2,2003
	Dallas FAA Office

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	All Others
	 
	 
	 
	 

	Transponder
	x
	334.2
	August 21,2003
	Certified August 21,2003

	Altimeter
	x
	334.2
	August 21,2003
	Certified August 21,2003

	Oil Change
	x
	335.4
	August 28,2003
	60 Weight Aero Shell

	        Oil Filter Drained
	x
	335.4
	August 28,2003
	Nothing found

	       Firewall Fuel filter drained and          Inspected 
	x
	335.4
	August 28,2003
	Nothing found

	Oil Change
	x
	351.2
	January 7,2004
	40 Weight Aero Shell

	Oil Change
	X
	356
	Sept 19, 2004
	50 Weight Aero Shell

	   Oil Filter Drained
	x
	356
	Sept 19, 2004
	Complete

	   Oil Cooler drained
	x
	356
	Sept 19, 2004
	Complete

	     Install Hobbs Meter
	x
	340.5
	October 20,2003
	Small panel at right side of Pilot 

	Install locks in rear cockpit
	x
	356.0
	Sept 19, 2004
	Lock down all switches controls etc.

	     Install Jacks for American Headsets
	x
	340.5
	October 20,2003
	Small panel at right side of Pilot and Co-Pilot

	 Installed YAK sack in rear  compartment
	 x
	 356.0
	 Sept 19, 2004
	 Complete

	 New Volt/Load meter
	 x
	 362.9
	 Oct 2, 2004
	 Installed and tested

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 

	 
	 
	 
	 
	 


