Nanchang CJ-6

Normal Procedures

Preflight

Walk Around Inspection.....…….……...................………………. Complete

Propeller (Mags Off!)………....…..….....….. Pull through 9 times (Min)

Oil Drain..............…………………………….………..…………………………Close

Rear Cockpit…...........………….……....……….… Secure (if no passenger)

Rear Mags............…………….……...…………...…………………………….. 1 + 2

Rear Gear Handle................….…………………..…… Centered & Latched

Rear Flap Handle..............……………...………….……....……………...Center

Failure Simulation Switches.............………………...…..……………...…Off

Rear Canopy….............…………………………….…………………………Closed

Before Start

Parachute and Harness.........................................................................Secure

Clock ................................................................................................................Wind

Main Air Valve..........… Full open then ½ turn Closed, Min 20 kg/cm2

Parking Brake...................................................................................................Set

Primer .................................................................................. Prime 5-7 strokes
    (Mags Off(!), preferred procedure 2 primes, 2 blades, repeat)

Throttle...................................................................................... Forward 1/3rd

Propellor............................................................................... High RPM (FWD)

Carb Heat.................................................................................................. Aft/Off

Emergency Fuel Cutoff............................................................ Full Forward

Master Power Switch…................................................................................On

Alternator Switch........................................................................................... On

Instrument Power Switch............................................................................On

Fuel Quantity...................................................................................... L/R Check

Wobble Pump ............................................ until fuel pressure indication

Emergency Air Valve................................................................................Closed

Engine Start

Propeller Area..............................................................................................Clear

Start Switch....................................................................................................... On

Mag Switch...................................................................................................... 1+2 
Start Button....................................................................................................Push

After Engine Start

Prime................................................. as necessary to keep engine running

RPM................................................................................................................. < 1000

Oil Pressure.................................................................................Min 1.5 kg/cm2

Fuel Pressure............................................................................................... Check

Volts/Amps.................................................................... Push (27v), Charging

Start Switch..........................................................................................................Off

Avionics Switch..................................................................................................On

12V Power Switch.............................................................................................On

GPS...........................................................................................................................On

Main Air Valve............……………………………...…………...… Confirm Open

Intercom Check....................................................................................Complete

Oil Cooler Door + Gills..................................................................As Required

Primer ................................................................................ Stowed and Locked
Radios/Comm/Nav/Altimeter/XPDR......................................................Set

Taxi

Air Pressure..................................................................................................Check

Brakes..............................................................................................................Check

Flight Controls............................................................................... Free & Clear

Engine Run-Up

Parking Brake.....................................................................................................Set

CHT Temp.............................................................................................. 120C Min

Oil Temp.................................................................................................... 40C Min

Engine RPM.....................................................................................................1860

Magneto Checks......................................................................... <75 Max drop

Engine RPM.....................................................................................................2100

Propeller...........................................................................................Cycle 3 times

Carb Heat........................................................................................................ Check

Throttle.................................................................................................................Idle

Flight Instruments.......................................................................................Check

Transponder........................................................................................................Alt

Passenger..................................................................................... Confirm Ready

Takeoff

Canopies............................................................................................ As Desired

Oil Temp...................................................................................................... > 40C

Cylinder Temp....................................................................................... > 120C

Oil Cooler Door + Gills...................................................................... Full Open

Trim............................................................................................ Set for Takeoff

Flaps................................................................................... Up/Check Indicator

Propeller....................................................................................... Full Forward

Throttle.........................................(mixture rich) Smoothly Full Forward

RPM................................................................................ 2350 (5 minutes max)

Rotate .......................................................................................................70 knots

Positive Rate of Climb.......................................................................Establish

Gear Safety Latch.....................................................................................Unlock

Gear Handle.............................................................................. Verify, then Up

Gear Lights..................................................................................... Verify 3 Red

Climb AirSpeed (Max Climb 80 knots)..................................... 100 knots

RPM............................................................................................. 2250 (16.5gph)

Mixture.......................................................................................................... Lean

Main Air Pressure.................................................................................... Check

Cruise

RPM..................75%(2250/660), 65%(1860/630), 55%(1790/600)

Mixture.............................................................................................................. Set

Engine Instruments....................................................................... Green Arc

Oil Cooler Door + Gills.........(Oil Out < 75, CHT<200)... As Required

Fuel Burn........................................................................ Monitor Even Burn

Descent

Air Vanes....................................................................................... As Required

Oil Cooler Door........................................................................... As Required

Landing

Prop Lever.................................................................................... Full Forward

RPM.............................................................................................................. 2000

Gear Handle......................................................... Down Below 130 knots

Gear Lights............................................................................................ 3 Green

Air Pressure.......................................................................................... Normal

Gear Indicator Pins........................................................................ Extended

Flap Handle............................................................. Down below 105 knots

Approach Speed................................................................................. 85 knots

Touchdown........................................................... 70 knots, stall (61 knots)

Braking........................................................................................... As Required

After Landing

Gear Safety Latch.....................................................................................Engage

Flap Handle................................................................................ Verify then Up

Transponder............................................................................................Standby

Oil Cooler Door + Gills............................................................................. Open

Engine Shutdown

Parking Brake....................................................................................................Set

Avionics Switch................................................................................................ Off

GPS......................................................................................................................... Off

Cylinder Temp.......................................................................................... < 150C

RPM............................................................................................. 1800 for 15secs

Throttle........................................................................................... Idle for 3secs

Mag Switch........................................................................................................... 0

12V Power Switch...........................................................................................Off

Instrument Power Switch............................................................................Off

Start Switch........................................................................................................Off

Alternator Switch.............................................................................................Off

Master Switch................................................................................................... Off

Flight Time......................................................................................................Note

Oil Cooler Door + Gills...............................................................................Close

Main Air Valve..............................................................................................Close

Emergency Egress Practice.................................................................Initiate

Parking/Securing

Controls/Gust Locks................................................... Secure (If on Ramp)

Canopies.........................................................................................................Close

Wheels......................................................................................................... Chock

Post Flight Walk-Around............................................................Accomplish

Snot Valve......................................................................................................Drain

Oil Collector Drain......................................................................................Open

Nanchang CJ-6

Non-Normal Procedures

Aerobatics

Harnesses...................................................................................................Secure

Canopies......................................................................................................Closed

RPM..................................................................................................................2250

Oil Cooler Door + Gills..............................................................................Open

Va...............................................................................................................149 knots

Vne............................................................................................................190 knots

Gmax................................................................................................. +6G’s -3 G’s

Go Around  (Memory Procedure)

Prop Lever.................................................................................. Full Forward

Throttle............................................. Smoothly Full Forward (2350rpm)

Positive Rate of Climb......................................................................Establish

Flap Handle.............................................................................. Verify, then Up

Gear Handle............................................................................. Verify, then Up

Climb Airspeed.....................................................................................90 knots

Touch and Go (Memory Procedure)

Aircraft.................................................................................. Positive Control

Flap Handle............................................................................. Verify, then Up

Prop Lever................................................................................. Full Forward

Throttle............................................. Smoothly Full Forward (2350rpm)

Positive Rate of Climb.....................................................................Establish

Gear Handle............................................................................ Verify, then Up

Climb Airspeed.....................................................................................90 knots

No Flap Landing

Approach Airspeed........................................................................ 100 knots

Touchdown Airspeed...................................................................... 80 knots

Alternator Failure

Alternator Switch...........................................................................................Off

12V Power Switch..........................................................................................Off

Nanchang CJ-6

Emergency Procedures

Engine Failure before Liftoff (Memory Procedure)

Throttle...........................................................................................................Idle

ATC.............................................................................................................Advise

Passenger.................................................................................................Advise

Aircraft.......................................................................... Exit Runway if Able

Parking Brake................................................................................................Set

Shutdown.......................................................................................Accomplish

Engine Failure during Initial Climb (Memory)

Throttle..........................................................................................................Idle

Airspeed.............................................................................................. 80 knots

Gear......................................... (If runway still available) Leave Down

Gear...................................................... (If no runway available) Retract

Fuel Cut Off Lever.............................................................................. Full Aft

Mags......................................................................................................................0

Canopies......................................................................................................Open

Flaps.............................................................................................................Down

Touchdown......................................................................................... 75 knots

Egress........................................................................................................Initiate

Engine Failure In Flight

Aircraft.................................................. Establish level upright attitude

Glide.............................................................................. Establish at 80 knots

Trim.................................................................................................... Best Glide

Landing Area............................................................. Locate Suitable Field

Throttle and Propeller........................................................... Full Forward

Mags............................................................................................................ 1 + 2

Wobble Pump......................................................................................... Pump

Fuel Pressure.......................................................................................Monitor

Primer................................................................................... Open and Pump

Starter........................................................................ Engage if no rotation

Transponder..............................................................................................7700

ATC..............................................................................................................Advise

Aircraft............................... If no start, prepare for Off Field Landing

Off Field Landing

Airspeed.............................................................................................. 80 knots

Propellor........................................................................ Full Aft (Feathered)

Fuel Cut Off Lever.............................................................................. Full Aft

Mags......................................................................................................................0

Canopies......................................................................................................Open

Gear................................................................................................ As Required

Flaps.............................................................................................................Down

Touchdown........................................................................................ 65 knots

Egress.......................................................................................................Initiate

Controlled Bail Out

Landing Area............................................................ Locate Suitable Field

Passenger.....................................................................................................Brief

Canopies.......................................................................................................Open

Parachutes............................................................................................ Tighten

Headsets............................................................................................... Remove

Trim........................................................................................ Full Nose Down

Attitude................................................................................. Roll to Inverted

Seat Belt................................................................................................. Release

Control Stick........................................................................................ Release

Once Clear of the Aircraft....................................................... D-Ring Pull

Emergency Bail Out (Memory Procedure)

Bail Out.......................... Command “Bail Out!” ”Bail Out!” Bail Out!”

Canopies......................................................................................................Open

Headsets............................................................................................... Remove

Seat Belt................................................................................................. Release

Egress...................................................................................................... Initiate

Once Clear of the Aircraft...................................................... D-Ring Pull

Alternate Gear Extension

Main Air Valve............................................................................................Close

Gear Handle(s)........................................................................................Center

Air Speed................................................................................. 105 knots Max

Emergency Air Valve..............................................................................Open

Gear Lights........................................................................................... 3 Green

Gear Indicator Pins........................................................................ Extended

Nanchang CJ-6

Limitations

Flight Conditions............................ Day/Night VFR and IFR if equipped

Max Weight............................................................................................. 3080lbs

Vne............................................................................................................ 201 knots

Va............................................................................................................... 149 knots

Vlo............................................................................................................. 130 knots

Vfe............................................................................................................. 108 knots

L/DMax.(Emergency Glide)..............................................................80 knots

Vs1............................................................................................................... 61 knots

Vso............................................................................................................... 59 knots

G-Limits........................................................................................................... +6 -3

Minimum Fuel for Aerobatics........................................................ 40 Liters

Unusable Fuel Qty............................................................................... 12 Liters

Minimum Oil Qty................................................................................    8 Liters

Oil Pressure...................................................................... Min 1.5, Normal 4-7

Oil Temp............................................................................................ Min IN>30C

Oil Temp.............................. Max Continuous IN<75C, 85C (15 minutes)

Oil Temp...........................................................................……. Max OUT=125C

CHT......................................... Min=120C, Normal 140C-210C, Max=230C

Mains 47psi, Nose 40psi

Nanchang CJ-6

Passenger Briefing Card

Welcome Aboard! The fact that you’re reading this means you are about to go for a ride in a piece of history.  The Nanchang first major production trainer used by China.  Over 2000 CJ-6s were produced between 1960 and 1985.  Produced in late October of 1979 this actually flew in China for 2500 hours, training their military pilots.  This CJ-6A arrived in the USA in 1995 and I purchased in in 2015; it had never been re-assembled.  I spent 3 years rebuilding it.

The CJ is a very capable aircraft. It has a 9 cylinder radial engine that produces 285hp at full power.  It can fly as high as 15,000’ above sea level and maintain a comfortable cruising speed of 150 knots.  Fuel burn is reasonable at 13 gallons per hour.  The aircraft was used for primary training, instrument training and aerobatic training.  The aircraft wing is designed to sustain 14 times the force of gravity although it is limited to no more than 7 g’s positive and 5 g’s negative in normal operation.

On your flight today you will experience the thrill and excitement of riding in a military trainer. Depending on what we have discussed on the ground, it may include aerobatic flight. Just remember, this is your ride. If you are not having fun than neither am I. Please let me know when you’ve had enough and we will come back to the airport.

Parachutes:  We wear parachutes as an added measure of safety and as a requirement of the FAA when performing aerobatic maneuvers or formation flight.  We do not anticipate using them but we are always prepared.  You will be briefed on their proper use and should you have any questions do not hesitate to ask.

The Canopy:  The canopy can be left open for all operations except aerobatics. The lever on the left side allows the canopy to be held open in one of several positions.  The ball hanging from the front of the canopy allows you to close it by reaching over your head and pulling down and forward.  Should it come open in flight please feel free to close it.  If you need additional air flow again feel free to set it anywhere you like. Just let me know what you are doing.

Safety Harness: The safety harness is a very simple device. It works by combining all of the 5 seat belts into one simple to close and open latch.  The 2 lap belts along with the center strap are what keep you in the seat.  The shoulder harness will keep you from hitting your head on the forward part of the cockpit.  Should you need to exit the aircraft simply move the silver lever to the left and release all of the belts at once.

Communication: You will be wearing a headset for better hearing. It is very important that we are able to communicate with each other; on the left side of the cockpit near your knee is the throttle. The top button is used to talk to the tower or other aircraft.  Do not use the top button unless you are instructed to do so.  It is also important that you do not restrict the movement of the throttle when in flight.  If we need to exit the aircraft remember to remove your headset.

Knobs, Dials and Switches:  The rear seat of the CJ was intended for the flight instructor.  With that in mind please realize that many of the controls you see in front and to the side of you are active.  They are not to be touched or moved without a specific instruction from me.  IMPORTANT!  Should you inadvertently move or touch something please say something!  It will not likely be a problem if I know about it. I will instruct you how to reset it.

Flying the CJ:  If you are asked to fly the aircraft do not be afraid. It is a relatively simple task to keep the aircraft flying safely.  The most important part is the transfer of control from the front seat to the rear seat.  It will happen in 3 distinct steps.

- I will tell you to take the controls by saying, “You have the aircraft”. As I do, I will shake the control stick. This is your signal to take the controls.

- You will then take the controls and say, “I have the aircraft.”

- At the same time you will shake the control stick from side to side so I can visually confirm that you indeed have the controls.

Now fly the aircraft as instructed by me and enjoy.  When you are done or I am ready to resume control we will simply use the procedure described above to return control of the aircraft back to me.

Not Feeling Well?  I do not carry sick sacks on board.  I expect you to let me know at the first indication that you are not feeling well so we can return to the airport.  The best remedy for airsickness is to open the canopy and get some fresh air.  At the same time look off to the left or right and find the horizon.  This should greatly reduce your queasiness.  Should you feel the need to vomit, please pull open your collar and vomit into your shirt.  This is the best way to contain the mess, and can easily be washed up.

Air Show Smoke: There may be times when I use the air show smoke system. It is a non-toxic smoke intended to mark the trail of the aircraft through the sky. The easiest way to remove the smoke from the cabin is to open the canopy just a bit. The air flow will clear the smoke quickly. If the smoke bothers you please let me know and I will stop using it. If you see or smell an acrid smoke or see any dark smoke in the cabin please say something immediately.

Bailing Out!!

It is never our intention to leave the aircraft while in flight. However, there may be an emergency situation where that is the safest course of action. Should we need to “Bail Out”, follow these instructions on my command.

When you hear

“Bail Out!” Bail Out!” Bail Out!”

Immediately:

- Remove your Headset

- Open the Canopy

- Release the Safety Harness (Red Belts)

- Exit the aircraft anyway you can

- When clear of the aircraft look for and pull the      D-Ring as previously instructed

- As you approach the ground, relax your body and roll off to one side as you touch down.

- Remain where you are.  I will come to you.

✈THANK YOU & HAVE A FUN FLIGHT!✈