PILOT OPERATING HANDBOOK (draft)

KOLB FIRESTAR II, Tandem

Registration No: 19-8829

Serial No: FSQ5-3-00079

BUILT BY STEVE GATES, 2018

SPECIFICATIONS

MAX TAKEOFF WT		330 kg		EMPTY WEIGHT		177 kg (incl unusable fuel)

MAX USEFUL LOAD		153 kg

CG LIMITS  (DATUM - wing leading edge)

- Most Forward: 			311 mm Aft of wing LE

- Most Aft:			544 mm Aft of wing LE

AIRSPEED (indicated)

- Never Exceed, Vne		90 mph

- Maneuvering, Va		75 mph

- Stall, Vs				27 mph (at full forward CG, engine idling)

- Best Glide Ratio, Vg		____

- Best Rate of Climb, Vy		____

- Best Angle of Climb, Vx		____

- Cruise				80 mph (70% power) 5,000 rpm

- Max Demonstrated Crosswind	____

AIRFRAME

- No of Seats		1-2

- Wingspan		27 ft		Length			22.5 ft

POWERPLANT/GEARBOX

- Manufacturer, model	Rotax 503 DCDI, electric start, oil injection

- Serial No		5766750	T.B.O.			300 hrs

- Power (max rated)	50 hp @ 6,500 rpm

- Fuel grades		91 RON min. automotive, EN Regular-Premium, AVGAS 100LL

- Fuel consumption	(@ Fuel density 0.72 Kg/L)

| Engine power |  |  | Speed | Specific fuel consumption |  | Fuel consumption |
| --- | --- | --- | --- | --- | --- | --- |
| % | hp | kW | rpm | g/kWh | L/kWh | L/h |
| 100 | 50 | 37.3 | 6500 | 500 | 0.694 | 25.9 |
| 85 | 42.5 | 31.7 | 5500 | 545 | 0.757 | 24.0 |
| 70 | 35 | 26.1 | 5000 | 600 | 0.833 | 21.8 |
| 50 | 25 | 18.7 | 4700 | 775 | 1.076 | 20.1 |

- Tot fuel capacity	2x5 gal = 37.8 L, Useable fuel ____ L

- Oil grade		Super two-stroke oil, Castrol Activ 2T or TTS

- Fuel-oil ratio		50:1 (injected)

- CHT (spark plug seat)	Normal 180-220 C (350-430 F)		Max 250 C (480 F)

- EGT 			Normal 460-580 C (860-1,000 F)		Max 650 C (1,200 F)

- Engine speeds:		Idle 		2,000 rpm

Continuous 	6,500 rpm

Max 		6,800 rpm (do not exceed)

- Gearbox		Type ‘B’, 2.58:1 reduction 		Oil: 85W-140EP

PROPELLER

- Manufacturer		Aerofibre/Bolly	Model 			Brolga

- Number of blades	3		Material		Composite

- Diameter		62 ins		Pitch			______

- Full throttle static 	____ RPM

PERFORMANCE

- Distance to Take-off & climb to 50 ft				_____

- Distance to Land over 50 ft obstacle to stop			_____

- Structural Limits 						+4 G	-2G

- Wing loading		5.9 lb/sq ft	Power loading 		14.5 lb/hp

PARACHUTE

- BRS model VLS800 	(800 lb aircraft), min safe deployment 75m AGL

NORMAL PROCEDURES

DAILY CHECKS

- Magnetos – OFF

- No water or contaminants in fuel/fuel filter

- Fuel vent tube unblocked

- Carb rubber socket & other rubber items for cracks & security

- Carb float chamber for water & dirt

- Air filters secure/lockwired

- Oil injection tank & hoses for leaks

- Ignition coils, wiring, spark plug leads

- Engine mounts & electric starter secure, cooling fan belt tight

- Throttle, Choke & oil pump cables secure & function

- Impulse fuel pump, impulse hose and all fuel tubing secure, not leaking, chafing or kinked

- 12V Battery secure

- Electric functions – EFIS, electric pump, strobe, 12V aux socket, radio PTT switch & headset

- Seatbelts secure, undamaged

- Documents in aircraft:

  - Pilot Operating Handbook

  - Weight & Balance w/equipment list

  - Repair & Alteration form

  - Aircraft Logbook & Engine Logbook (available)

- Gearbox drain & level plugs safety wired

- Spark plugs & wires secure

- VERIFY MAGS OFF - Rotate/rock prop by hand – check unusual noises & bearings slop

- Propeller undamaged, bolts secure

- Exhaust, springs/lockwires & temp probes secure

- Wing struts pins & bolts secure, safety pins all ends, no cracks or corrosion.

- Wing drag struts bolts secure, no cracks or corrosion.

- Wings & fabric undamaged

- Aileron control linkages secure, free & correct, hinges secure/lockwire

- Elevator control cables secure, free & correct

- Rudder control cables secure, free & correct

- Brakes free, and locking when depressed

- Tail boom tube attachment to aft fuselage – no cracks, kinks or corrosion

- Horizontal stabiliser, fabric & hinges secure/lockwire

- Vertical stabiliser fabric & hinges secure/lockwire

- Empennage guy wires tight & undamaged/corroded. Bolts secure (Bottom w/safety pin).

- Tailwheel, pivot & rudder connection

- Main wheels secure, inflated, brake cables & end fittings secure

- Pitot tube secure & unblocked. Static port (under instrument panel) unblocked.

PREFLIGHT CHECKS

- Weight & Balance

- Magnetos – OFF

- Fuel quantity/mixture (premixed and/or injected), contaminants, fuel caps secure

- Oil injection quantity

- Air filters secure/lockwired, remove covers

- Spark plug leads

- Impulse fuel pump, impulse hose and all fuel tubing secure, not leaking, chafing or kinked

- Gearbox leaks

- Propeller undamaged

- Exhaust, springs/lockwires & temp probes secure, remove exhaust plug

- Wing struts pins & bolts secure, safety pins all ends

- Wing drag struts bolts secure

- Wings & fabric undamaged

- Tail boom tube attachment to aft fuselage – no cracks, kinks or corrosion

- Horizontal stabiliser, fabric & hinges secure/lockwire

- Vertical stabiliser fabric & hinges secure/lockwire

- Empennage guy wires tight & undamaged/corroded. Bolts secure (Bottom w/safety pin).

- Tailwheel, pivot & rudder connection

- Pitot tube cover removed & Static port unblocked

- Ballistic parachute safety pin removed

- Wing tie-downs removed and secured

PRE-TAKEOFF CHECKS & ENGINE START

- Flight controls free & correct

- Circuit breakers (panel) IN

- Master switch ON

- Electric fuel pump ON

- Choke ON

- Magnetos ON

- Throttle IDLE

- Brakes ON

- CLEAR PROP - Engine start then throttle to LOW, Choke OFF

- Engine warm-up - 2,000 rpm for 2 mins, then 2,500 rpm until normal CHT temp: 180-220C (350-430F)

- EFIS ON

- Instruments settings/readings - Altimeter, Compass, GPS, OAT

- Gauges – CHT & EGT normal, Battery charging (Note: If not charging, check circuit breaker red L.E.D. in Electric junction box under rear seat. If lighted, then wait 30 sec for auto re-set)

- Radio/intercom ON

- Strobe ON

- Magnetos CHECK at 3,000 – 3,500 rpm (300 rpm max. allowable drop for either. WARNING:  If no drop, then magneto grounding or timing may be FAULTY)

- Full throttle response briefly to 6,500 rpm

- Seatbelts SECURE

TAKE-OFF & CLIMB

- Brakes OFF

- Throttle FULL 6,500 rpm (to avoid lean mixture),

- Liftoff 40 mph (TBC)

- Climb 50 mph (TBC)

Note: 	Any exceedance of engine speed, CHT or EGT - enter in logbook value and duration

IN-FLIGHT & DESCENT

- Do not exceed max rpm

- Avoid lean condition with high rpm and low throttle opening, causing high temps.

- Electric fuel pump OFF

LANDING

- Seat belts SECURE

- Electric fuel pump ON

- Approach airspeed 40mph

- Avoid lean condition with high rpm and low throttle opening, causing high temps.

- Brakes OFF

SHUTDOWN

- Run engine at 3,000 rpm for 2 mins, then 2,000 rpm short idle

- Electric fuel pump OFF

- Choke ON momentarily, then Mags OFF

- Radio OFF

- Engine Hobbs time RECORD

- EFIS, Strobe & Master OFF

- Ballistic Recovery chute safety pin INSTALL

- Pitot cover, fuel vent line cover, carb covers, plug exhaust INSTALL

EMERGENCY PROCEDURES

ENGINE FAILURES

During Takeoff Roll

- Throttle  IDLE

- Brakes  APPLY

- Magnetos  OFF

- Master  OFF

After Takeoff

- Airspeed  ____ mph (Vg)

- Electric fuel pump OFF

- Magnetos OFF

- Land STRAIGHT AHEAD

- Master OFF

In flight & Engine Restart (may be futile – set up for landing before attempting this)

- Airspeed ____ mph (Vg)

- Prepare for landing

- Choke ON (if engine not warm)

- Electric Fuel pump ON

- Starter START

- Choke OFF

- If restart not successful:

  - Fuel pump OFF

  - Magnetos OFF

  - Land OR deploy Ballistic parachute

  - Master OFF

CARBURETTOR ICING

- Reduce throttle and/or throttle up/down to attempt to dislodge ice

- Descend to warmer air

FIRE

On ground:

- Electric Fuel pump OFF

- Magnetos OFF

- Master OFF

In-flight:

- Fuel pump OFF

- Magnetos OFF??

- Master OFF

- Maximise airspeed in glide to attempt to extinguish flames

- Land OR deploy Ballistic parachute (pull red trigger handle in front of joystick)

SPIN RECOVERY (inadvertent)

- Ailerons NEUTRAL

- Throttle IDLE

- Rudder FULL OPPOSITE DIRECTION OF SPIN

- Elevator DOWN BRISKLY ENOUGH TO BREAK STALL

- Hold inputs until rotation stops

- Recover NEUTRALISE RUDDER, ELEVATOR UP FOR GENTLE RECOVERY

ENGINE PRESERVATION

ENGINE STOPPAGE 1-4 WEEKS

- With warm engine, idle at increased idle speed

- Shut down engine

- Remove air filters & inject approx. 3cc preservation oil or equivalent into each carburetor

- Restart engine and run at increased idle speed for 10-15 seconds

- Shut down engine

- Close all engine openings (exhaust, vent tube, air filters) against moisture and contaminants

ENGINE STOPPAGE 4 WEEKS – 1 YEAR

- With warm engine, idle at increased idle speed

- Remove air filters & inject approx. 6cc preservation oil or equivalent into each carburetor

- Shut down engine

- Remove plugs & inject approx. 6cc preservation oil or equivalent into each cylinder, and slowly turn crankshaft 2-3 turns by hand to lubricate top end parts.

- Lubricate all carburettor linkages

- Close all engine openings (exhaust, vent tube, air filters) against moisture and contaminants

- Spray all external steel parts with engine oil

TROUBLESHOOTING

| Problem | Check/Remedy |
| --- | --- |
| Fuel issues | Check fuel supply, fittings, filter, float chamber |
| Ignition issues | Try new plugs |
| Engine runs with mags off | Possibly due to overheating: let engine cool down at idle at approx. 2,000 rpm. Check mags inadvertently grounded (chafed wiring, etc) |
| Knocking under load | Octane rating of fuel too low: use fuel with higher Octane rating. Fuel starvation/lean mixture: check fuel supply |
| Other | Recommend Rotax assistance: Bert Flood Imports, Lilydale, VIC. Ph: 03 9735-5655 |